American International University-Bangladesh

Faculty of Engineering

Department of Electrical and Electronic Engineering

| Final Assignment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Course Name: | Microprocessor and Embedded Systems with Lab |  |  | Course Code: |  | EEE 4103 |
| Semester: | Summer 2025-26 |  |  | Section: |  |  |
| Faculty Name: | Prof. Dr. Engr. Muhibul Haque Bhuyan |  |  |  |  |  |
| Submission Date: | ____/05/2026 | Assigned Date: | 10/08/2026 | Due Date: | 02/09/2026 |  |

Student Information:

| Group #: 0___ | Student ID # |  |  |  |  |  |  |  |  |  | Department | Signature |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Student Name | p | q | - | a | b | c | d | e | - | r |  |  |

Special Instruction:

- Questions may be copied from here through copy-paste. Online submission via TEAMS is allowed. However, hard copy must be submitted to the course teacher.

- Group-wise submission consisting of 20 marks for each member of the group, submit 1 copy per group.

- Please read the assessment rubrics carefully.

Assessment Rubrics:

| COs-POIs | Excellent [26-30] | Proficient [21-25] | Good [16-20] | Acceptable [9-15] | Unacceptable [1-8] | No Response [0] | Secured Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO4 P.a.4.C.3 | All the problems are solved correctly. The simulation processes are clearly described, and results are generated by combining all possible input patterns with appropriate outcomes. All necessary drawings and computations are shown correctly. | All the problems are solved correctly. The simulation processes are clearly described, and results are generated by combining all possible input patterns with appropriate outcomes. A few necessary drawings and computations are missing but no wrong drawing. | All the problems are solved correctly. The simulation processes are not clearly described, and results are generated by combining all possible input patterns with appropriate outcomes. A few necessary drawings and computations are missing or wrong. | All the problems are not solved correctly. The simulation processes are not clearly described, and results are generated by combining several wrong or less input patterns with in/appropriate outcomes. Some necessary drawings and computations are missing or wrong. | All the problems are not solved correctly. The simulation processes are not described, and results are generated by combining mostly wrong input patterns with inappropriate outcomes. Almost all the necessary drawings and computations are missing or wrong. | No responses at all or copied from others |  |
| Comments |  |  |  |  | Total Marks (30) |  |  |

- Compute the baud rate for the asynchronous normal operating mode when the oscillator frequency, fOSC  = pe MHz, and register data is, UBRRn = 110100101011. Calculate the baud error and comment on whether there will be any communication errors or not. Standard Baud rates are: 300, 600, 1200, 2400, 4800, 9600, 14400,19200, 38400, 57600, 115200, 230400, ... bps.

- Compute the baud rate for the synchronous master operating mode when the oscillator frequency, fOSC  = qd MHz, and register data is, UBRRn = 011100101011. Calculate the baud error and comment on whether there will be any communication error or not. Standard Baud rates are: 300, 600, 1200, 2400, 4800, 9600, 14400,19200, 38400, 57600, 115200, 230400, ... bps.

- Compute the baud rate for the asynchronous double-speed operating mode when the oscillator frequency, fOSC  = rc MHz, and register data is, UBRRn = 010110101110. Compute the baud error and comment on whether there will be any communication errors or not. Standard Baud rates are 300, 600, 1200, 2400, 4800, 9600, 14400,19200, 38400, 57600, 115200, 230400, ... bps.

- Compute the travel time of the sonar wave as per the following program codes of Arduino if the object is ab.qp cm away from the sonar sensor. Determine the number of LEDs that are turned ON. To display a message for any distance that how many LEDs are turned ON/OFF (for example, “Distance = xx cm;” in one line and in the next line “nn LEDs are turned ON and n LED is turned OFF”) then write the necessary changes that are required to be included in the following program. Draw the relevant circuit diagram using Proteus and show its simulation results including the serial monitor’s outputs (full screen shot of your computer).

const int trigPin = 8; // define the pin numbers

const int echoPin = 9; // define the pin numbers

long duration; // define variables

float distance, distanceinches, distanceThreshold; // define variables

void setup() {

Serial.begin(57600); // Starts the serial communication.

pinMode(trigPin, OUTPUT); // Sets the trigPin as an Output

pinMode(echoPin, INPUT); // Sets the echoPin as an Input

pinMode(2, OUTPUT); // Sets pins 2, 3, and 4 as the Output pin

pinMode(3, OUTPUT);

pinMode(4, OUTPUT);

}

void loop() {

digitalWrite(trigPin, LOW);   // Clears the trigPin

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);   // Sets the trigPin on HIGH state for 10 microseconds

digitalWrite(trigPin, LOW);

// Reads the echoPin, returns the sound wave travel time in microseconds

duration = pulseIn(echoPin, HIGH);

distance = (duration/2) * 1e-6 * 340 * 100;   // Calculating the distance

distanceinches = (distance / 2.54);

Serial.print("Distance = ");   // Prints the distance on the Serial Monitor

Serial.print(distance);

Serial.print(“ cm; ”);

distanceThreshold = 100;   // set threshold distance to activate LEDs

if (distance > distanceThreshold) {

digitalWrite(2, LOW);

digitalWrite(3, LOW);

digitalWrite(4, LOW); }

if (distance < distanceThreshold && distance > distanceThreshold-30) {

digitalWrite(2, HIGH);

digitalWrite(3, LOW);

digitalWrite(4, LOW); }

if (distance < distanceThreshold-30 && distance > distanceThreshold-50) {

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, LOW); }

if (distance < distanceThreshold-50 && distance > distanceThreshold-70) {

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH); }

delay(300); // Wait for 300 millisecond(s)

}

- Compute the duty cycle and sketch the waveform obtained at port D of the Arduino. Identify the modes of operation and compute the operating frequency of that mode based on the following program segment. Identify the Timer of the Arduino Microcontroller. The system clock frequency is pc MHz. Draw the relevant circuit diagram using Proteus and show its timing diagram (full screen shot of your computer).

DDRD |= (1<<PD5);

pinMode(5, OUTPUT);

OCR0B= (153+0+a+b); // Load OCR0B for setting its duty cycle

// Configure TCCR0A and TCCR0B registers for the mode and pre-scaler

TCCR0A |= (1 << COM0B1) | (1<<WGM01) | (1<<WGM00);

TCCR0B |= (1<<CS02) | (1<<CS00);

- Compute the duty cycle and sketch the waveform obtained at port D of the Arduino. Identify the modes of operation and compute the operating frequency of that mode based on the following program segment. Identify the Timer of the Arduino Microcontroller. The system clock frequency is re MHz. Draw the relevant circuit diagram using Proteus and show its timing diagram (full screen shot of your computer).

DDRD |= (1<<PD5);

pinMode(5, OUTPUT);

OCR0A = (200+ a + b + c);  // Load a value in the OCR0A register

OCR0B= (100 + d + e);  // Load a value in the OCR0B register

// Configure TCCR0A and TCCR0B registers for the mode and pre-scaler

TCCR0A |= (1 << COM0B1) | (1 << COM0A0) | (1<<WGM01) | (1<<WGM00);

TCCR0B |= (1<<WGM02) | (1<<CS01) | (1<<CS00);

- Develop the control words in binary and hexadecimal formats (the necessary bits for the control word are given in Table 2) using the information provided in Table 1 for the following micro-operations:

| - Re←Ra+Rb | - Rd←3(Re - 0)/3 | - Rq←Rp-Ra | - Rq←Input |
| --- | --- | --- | --- |
| - Rq←SHL Rp | - Output←Rc | - Rc←CRC Rd | - Rr←SHR Rb |
| - Rd←Rc | - Rb←0 | - Rp←RaURd | - Rr←SHR RaURd |

* If any value of a-e goes above 7 then it should be assumed as 7.

Table 1: Functions of control variables

| Binary Code | Functions of selection variables |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A | B | D | F with Cin = 0 | F with Cin = 1 | H |
| 0 0 0 | Input Data | Input Data | None | A-1 | A | 1’s to the output Bus |
| 0 0 1 | R1 | R1 | R1 | A+B | A+B+1 | Shift Left with IL = 0 |
| 0 1 0 | R2 | R2 | R2 | A-B-1 | A-B | No Shift |
| 0 1 1 | R3 | R3 | R3 | A | A+1 | Circulate Left with Carry |
| 1 0 0 | R4 | R4 | R4 | A | X | 0’s to the output Bus |
| 1 0 1 | R5 | R5 | R5 | AXOR B | X | 0’s & 1’s to the lower and upper nibbles |
| 1 1 0 | R6 | R6 | R6 | A AND B | X | Circulate-Right with Carry |
| 1 1 1 | R7 | R7 | R7 | A OR B | X | Shift Right with IR = 0 |

Table 2: 16-bit control word sequence

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  |  | B |  |  | D |  |  | F |  |  | Cin | H |  |  |

- Develop the control memory outputs for the sequence in Table 3 using the information listed in Table 1. To complete the memory outputs, use the microinstructions that you have developed in question no. 7.

Table 3: Control memory bit sequence

|  |  |  | ROM outputs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROM Address |  |  | Control Word |  |  |  |  |  |  |  |  | Address |  |  | Mux Select |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Determine the contents of the various registers and various buses and flags after the next clock cycle of Fig. 1 as per Table 4 when the ROM addresses are r, r+1, r+2, r+3, r+4, r+5, and r+6 (written in 6-bit binary). Determine the contents of the ROM address locations as per Table 3, if necessary, draw more rows.

Fig. 1 Micro-program control for processor unit with instructions.

Table 4: Control memory bit sequence

|  |  |  |  |  |  | ROM outputs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROM Address |  |  |  |  |  | A |  |  | B |  |  | D |  |  | F |  |  | Cin | H |  |  | Mux Select |  |  |  | Address |  |  |  |  |  |
|  |  |  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Show the flow chart and develop the micro-instruction table for counting the number of 1’s in register, Ra, and storing the count in register Rb. Assume that the starting address is r (written in 6-bit binary). The operation should follow the control word format of Table 2. If register, Ra contains 10111001, determine the contents of register, Rb after the micro-operation is completed. Control memory structure is given in Fig. 1.

* If any value of a-e goes above 7 then it should be assumed as 7.

Fig. 2 Micro-program control for processor unit.